Threat and Risk Assessment for Passenger Information System

Document Version: V1.0

Assessment Date: 2026-03-03

Assessment Team: Bob

Report Review: Tom, Alex

Issue Date: 2026-03-03

1. Introduction

1.1 Assessment Background

The Passenger Information System (hereinafter referred to as the “PIS”) that developed and manufactured by the company is mainly used to display core functions such as station information display, real-time passenger counting, station arrival reminders, and passenger seat reservations. It is critical infrastructure ensuring transportation operation efficiency and improving passenger travel experience.

The system includes dedicated hardware equipped with one Ethernet interface and one USB interface, named PIS controller. It communicates with other system modules via the TCP/Bluetooth/OPCUA/MODUBUS protocol. Data inside the controller is saved in plain text.

To comprehensively identify security threats to the PIS, quantify risk levels, and formulate targeted security control measures, this Threat and Risk Assessment (TRA) is conducted.

1.2 Assessment Objectives

- Identify core assets, system boundaries, and operating environments of the PIS, establish a complete asset list, and lay the foundation for risk assessment.

- Comprehensively identify security threats to the PIS in hardware, software, data, communication, management, and other dimensions, and analyze threat sources and trigger conditions.

- Develop scientific, feasible, and implementable security control measures and emergency response plans for risks of different levels to reduce risks to an acceptable level.

1.3 Assessment Scope

1.3.1 System Scope

This assessment covers the life cycle of the PIS developed by the company, including hardware equipment, software components, data resources, communication links, and operation and management links, as detailed below:

- Hardware equipment: dedicated PIS terminals (including network interface and USB interface), data transmission modules, display and audio terminals, etc.

- Software components: system firmware, data processing programs, communication protocol stack, seat reservation module, display control module, etc.

- Data resources: runtime data (real-time passenger count, vehicle status, station arrival information, etc.) and non-runtime data (device configuration, logs, seat reservation records, device parameters, etc.).

- Communication links: TCP-based communication links between PIS terminals and other system modules, and data transmission channels of device interfaces (network port, USB).

- Management links: daily management processes such as device deployment, operation and maintenance, permission management, firmware upgrade, and data backup.

1.3.2 Assessment Boundary

This assessment does not include the internal security of external third-party systems connected to the PIS (such as traffic dispatch platforms and ticketing systems), but only evaluates the security risks of the PIS itself and its external interfaces. It does not involve physical damage to hardware (such as equipment failures caused by man-made damage or natural disasters), but only focuses on security threats at the device interface and logical levels.

1.4 Assessment Methodology

This assessment adopts the standard TRA methodology, combined with the industry and technical characteristics of the PIS. The following methods are used to ensure comprehensive and accurate results:

- Asset identification: sort out core assets, clarify asset value and importance through document review, technical disclosure, and on-site investigation.

- Threat identification: identify threats through vulnerability scanning, manual penetration testing, threat intelligence analysis, and expert judgment.

- Impact analysis: analyze direct and indirect impacts on assets, business operations, passenger rights, and corporate reputation, and quantify impact levels.

- Likelihood analysis: analyze the probability of threats based on threat sources, trigger conditions, and current protection status.

- Risk rating: use a quantitative model Risk Level = Likelihood × Impact to classify risks as Critical, High, Medium, and Low.

2. System Overview

2.1 System Architecture

The PIS adopts a distributed architecture, consisting of four layers: terminal layer, communication layer, data layer, and application layer. They work together to realize the collection, processing, transmission, and display of passenger information.

- Terminal layer: includes dedicated PIS hardware terminals and display terminals. Terminals are equipped with one network port (for communication) and one USB interface (for debugging, firmware upgrade, or data export). They collect real-time passenger count and receive station information, and display content to passengers.

- Communication layer: uses TCP as the core protocol for data transmission between PIS terminals and other modules. Runtime data is encrypted; non-runtime data is transmitted in plaintext.

- Data layer: stores runtime data (encrypted) and non-runtime data (plaintext), including passenger flow, vehicle status, configuration, logs, and reservation records.

- Application layer: includes seat reservation, station reminder, passenger flow statistics, and display control modules. It processes data collected by terminals and issues control commands.

2.2 Core Functions

- Passenger flow display: collects real-time passenger count and displays occupancy status.

- Station arrival reminder: receives dispatch information and reminds passengers via voice and text.

- Seat reservation: supports seat booking, records information, and displays occupancy status.

- Device management: allows O&M personnel to debug, upgrade firmware, and configure parameters via network or USB.

- Log recording: records system status, terminal operations, and data transmission for O&M and troubleshooting.

2.3 Current Security Status

The PIS has adopted basic security measures but still has obvious shortcomings:

- Data security: runtime data is encrypted in transmission and storage; non-runtime data is unencrypted, risking leakage and tampering.

- Communication security: TCP is used; only runtime data is encrypted. Lack of two-way authentication risks identity forgery and data hijacking.

- Device security: network and USB interfaces lack strict access control, risking malicious access and tampering.

- O&M security: lack of complete operation audit; no full traceability, leading to repudiation risks.

- Terminal security: embedded terminals have limited performance, no terminal protection software, and weak resistance to DoS and privilege escalation attacks.

3. Asset Identification and Classification

Assets are the core of PIS protection. Assets are classified into three levels based on importance, value, and impact:

- Level 1: Core Assets

- Level 2: Important Assets

- Level 3: General Assets

3.1 Asset List

表格

| Asset ID | Asset Type | Asset Description | Asset Value | Importance Level | Remarks |
| --- | --- | --- | --- | --- | --- |
| ASSET-001 | Hardware | Dedicated PIS terminal (with network & USB) | High | Level 1 | Core terminal for data collection & display |
| ASSET-002 | Hardware | Display terminal | Medium | Level 2 | Affects passenger experience |
| ASSET-003 | Data | Runtime data (passenger flow, status, stations) | High | Level 1 | Encrypted, supports core business |
| ASSET-004 | Data | Non-runtime data (config, logs, reservations) | High | Level 1 | Plaintext, contains privacy & configuration |
| ASSET-005 | Software | System firmware, data processing programs | High | Level 1 | Vulnerabilities may lead to attacks |
| ASSET-006 | Software | Communication protocol stack (TCP) | Medium | Level 2 | Data transmission, has inherent risks |
| ASSET-007 | Application | Seat reservation & station reminder modules | Medium | Level 2 | Directly affects experience & operation |
| ASSET-008 | Communication | Network port, USB interface, TCP links | High | Level 1 | Transmission channel, common attack entry |

3.2 Asset Classification Description

- Level 1 (Core Assets): PIS terminal, runtime/non-runtime data, firmware, communication interfaces. Directly related to operational safety and privacy.

- Level 2 (Important Assets): Display terminal, protocol stack, core application modules. Affect experience and efficiency but not system paralysis.

- Level 3 (General Assets): Auxiliary equipment (e.g., power adapters). Minimal impact on core functions.

4. Threat Identification and Analysis

4.1 Threat Classification

Threats are divided into five categories:

- Hardware interface threats

- Data security threats

- Communication security threats

- Software security threats

- Management security threats

4.2 Detailed Threat Identification and Analysis

表格

| Threat ID | Threat Category | Threat Description | Threat Source | Trigger Condition | Affected Assets |
| --- | --- | --- | --- | --- | --- |
| THREAT-001 | Hardware Interface | Malicious USB access, firmware tampering, data export | External attacker, malicious O&M | No USB access control; no device verification | ASSET-001, 004, 005 |
| THREAT-002 | Hardware Interface | Malicious network access, impersonate legitimate terminal | External attacker, internal malicious user | No network access control; no authentication | ASSET-001, 003, 004, 008 |
| THREAT-003 | Data Security | Non-runtime data leakage in plaintext | Attacker, sniffer | Unencrypted non-runtime data | ASSET-004 |
| THREAT-004 | Data Security | Tampering with non-runtime data or configuration frames | Attacker, malicious O&M | No integrity check or signature | ASSET-001, 003, 004 |
| THREAT-005 | Data Security | Log deletion or tampering | Attacker, malicious O&M | No log encryption or anti-tampering | ASSET-004 |
| THREAT-006 | Communication Security | Sniffing TCP links, decrypting runtime data | Attacker, sniffer | Weak encryption, poor key management | ASSET-003, 008 |
| THREAT-007 | Communication Security | Forged TCP identity, injecting false data | Attacker | No two-way authentication | ASSET-001, 003, 007, 008 |
| THREAT-008 | Communication Security | TCP flood, DoS attack | Attacker | No rate limiting; weak terminal performance | ASSET-001, 003, 007, 008 |
| THREAT-009 | Software Security | Exploit firmware vulnerabilities to gain root access | Attacker | Unpatched vulnerabilities; no upgrade verification | ASSET-001, 005 |
| THREAT-010 | Software Security | Tamper firmware, implant malware | Attacker, malicious O&M | No integrity check; unsecure upgrade | ASSET-001, 003, 004, 005 |
| THREAT-011 | Management Security | Unreasonable permission assignment, over privilege | Malicious O&M | No least privilege; no audit | ASSET-001, 004, 005 |
| THREAT-012 | Management Security | No O&M operation audit, unable to trace responsibility | O&M personnel, attacker | No audit system; no log retention | All Level 1 & 2 assets |

5. Risk Rating and Analysis

5.1 Risk Rating Criteria

5.1.1 Likelihood Level (1–5 points)

表格

| Likelihood | Score | Description |
| --- | --- | --- |
| Very High | 5 | Easy to trigger, high probability |
| High | 4 | Relatively easy to trigger |
| Medium | 3 | Moderate difficulty and probability |
| Low | 2 | Complex conditions, low probability |
| Very Low | 1 | Extremely difficult, nearly impossible |

5.1.2 Impact Level (1–5 points)

表格

| Impact | Score | Description |
| --- | --- | --- |
| Very High | 5 | System paralysis, major accident, heavy reputation damage |
| High | 4 | Partial failure, data compromise, operational disruption |
| Medium | 3 | Partial abnormality, minor impact |
| Low | 2 | Slight local impact |
| Very Low | 1 | No actual impact |

5.1.3 Risk Level (Total = Likelihood × Impact)

表格

| Risk Level | Score Range | Handling Requirement |
| --- | --- | --- |
| Critical | 20–25 | Immediate remediation |
| High | 12–19 | Priority remediation |
| Medium | 6–11 | Routine management |
| Low | 1–5 | Monitoring only |

5.2 Risk Rating Results

表格

| Threat ID | Threat Description | Likelihood | Impact | Total | Risk Level | Priority |
| --- | --- | --- | --- | --- | --- | --- |
| THREAT-001 | USB malicious access, tampering, data leakage | 4 | 5 | 20 | Critical | P0 |
| THREAT-002 | Malicious network access, impersonation | 4 | 4 | 16 | High | P1 |
| THREAT-003 | Non-runtime data plaintext leakage | 5 | 5 | 25 | Critical | P0 |
| THREAT-004 | Tampering non-runtime data / configuration frames | 5 | 4 | 20 | Critical | P0 |
| THREAT-005 | Log deletion/tampering | 4 | 4 | 16 | High | P1 |
| THREAT-006 | TCP sniffing, data decryption | 3 | 4 | 12 | High | P1 |
| THREAT-007 | Identity forgery, false data injection | 3 | 4 | 12 | High | P1 |
| THREAT-008 | TCP flood, DoS | 4 | 3 | 12 | High | P1 |
| THREAT-009 | Firmware vulnerability exploitation, root access | 3 | 5 | 15 | High | P1 |
| THREAT-010 | Firmware tampering, malware implantation | 3 | 5 | 15 | High | P1 |
| THREAT-011 | Unreasonable permission assignment | 3 | 4 | 12 | High | P1 |
| THREAT-012 | Lack of O&M audit | 3 | 4 | 12 | High | P1 |

5.3 Risk Summary and Analysis

5.3.1 Overall Risk Distribution

A total of 12 threats were identified:

- 3 Critical risks (25%)

- 9 High risks (75%)

- No Medium or Low risks

The overall security risk level of the PIS is high.

5.3.2 Core Risk Analysis

Core risks focus on three areas:

- Non-runtime data security: unencrypted storage and transmission lead to extremely high leakage and tampering risks.

- Hardware interface security: no access control or authentication on USB and network ports, which are major attack entry points.

- Communication and software security: lack of two-way authentication and rate limiting; firmware vulnerabilities expose terminals to takeover.

5.3.3 Risk Impact Summary

Impacts cover four dimensions:

- Operational safety: system failure, disorder of transportation operations.

- Passenger rights: privacy leakage, misleading information, complaints.

- Compliance: violation of Data Security Law and Personal Information Protection Law, penalties.

- Technical security: terminal takeover, data breach, potential lateral movement.

6. Security Control Measures

6.1 Critical Risk (P0) Control Measures

表格

| Threat ID | Control Measures | Responsible Department | Implementation Period | Verification Method |
| --- | --- | --- | --- | --- |
| THREAT-001 | 1. Enable USB access control with whitelist.2. Disable USB storage, retain only debugging with authentication.3. Enable USB access audit.4. Add physical protection for USB ports. | R&D Dept., O&M Dept. | Within 1 month | Test unauthorized access rejection; check audit logs; verify physical protection. |
| THREAT-003 | 1. Encrypt all non-runtime data with AES-256.2. Establish key management system.3. Use encrypted channels for non-runtime data.4. Regularly check encryption status. | R&D Dept., Security Dept. | Within 1.5 months | Packet capture verification; check storage encryption; review key management. |
| THREAT-004 | 1. Add SHA-256 integrity check and digital signature.2. Encrypt and sign configuration frames.3. Verify data before processing.4. Establish anomaly detection and alarm. | R&D Dept., Security Dept. | Within 1.5 months | Test tampering resistance; verify encryption & signature; check alarm function. |

6.2 High Risk (P1) Control Measures